Contents

1. Forces in Equilibrium	1

Objectives	1

Materials and Equipment	1

Safety	1

Procedure	1

Part 1: Loads	1

Part 2: Reaction Forces	2

Data Collection	2

Questions and Analysis	3

# 1. Forces in Equilibrium

What forces are experienced by a bridge?

To design well-built bridge, you must understand the forces the bridge will be under. The two basic forces experienced by a bridge are loads and reaction forces. To measure these forces in a model, we will use a sensor called a Load Cell.

## Objectives

- Identify loads and reaction forces, including indicating the direction of the force.

- Calculate unknown forces using the concept of equilibrium of forces.

## Materials and Equipment

- Data collection system

- Building Better Bridges Kit

## Safety

Follow regular laboratory safety precautions.

## Procedure

### Part 1: Loads

- Select Sensor Data in SPARKvue and connect your load cell to your computing device.

- From Quick Start Experiments, select Load Cell Force.

- Insert a screw on each side of the load cell.

- Place all available washers on the weight hanger.

- Hold the load cell in one hand. Zero the sensor and start collecting data.

- Hang the weights on the fixed side of the load cell. See Figure 1.

Figure 1: Weights on load cell

- Stop recording data and record the force in Table 1.

- Repeat steps 5 - 7 for the Load side of the sensor. You will learn about positive and negative forces in the next lesson. For this activity record your data in positive integers.

### Part 2: Reaction Forces

- Build a simple bridge consisting of a single beam supported on each end. Attach a beam to each side of the load cell and replace one leg of the bridge with the load cell. See Figure 2.

Figure 2: Load cell attached to bridge

- On the diagram below, enter the load force from Part 1 onto the top of each Case. Remember for this activity only use positive integers.

- Zero out the sensor and start collecting data.

- Hang the weight from the center of the beam. The weight and legs of the bridge are represented as a 2D diagram in Case A below. Record the weight in the diagram below.

- Repeat steps 3 - 4 for Case B and Case C. Note the placement of the weights in each of these cases.

- Calculate the sum of the support forces in the legs for each case.

## Data Collection

Table 1: Force Measured by Load Cell

| FIXED (N) | LOAD (N) |
| --- | --- |

Figure 3: Sample Case Answers

## Questions and Analysis

- When measuring the weight of a load, should you hold the weight from the load or fixed side? Using data, explain.

- Why is it important to zero the load cell before taking your measurements?

- Using data from Part 2, how does the sum of the forces compare to the weight of the load?

- Propose an explanation for why the weight is represented by a downward arrow and why the legs are represented by upward arrows in your diagram.

- In which case is each support force equal and different from each other? Propose and explanation.