Contents

2. Internal Forces	0

Objectives	0

Materials and Equipment	0

Safety	0

Procedure	0

Part 1: Measuring Tension and Compression	0

Part 2: Tension and Compression in Trusses	0

Data Collection	0

Questions and Analysis	0

2. Internal Forces	0

Procedural Overview	0

Time Requirements	0

Objectives	0

Materials and Equipment	0

Teacher Tips	0

Safety	0

Data Collection	0

Questions and Analysis	0

Technical Support	0

# 2. Internal Forces

How are forces distributed within a truss when a load is applied?

## Objectives

- Explain how the force on a truss joint is related to the force in a truss member.

- Resolve diagonal forces into vertical and horizontal components.

- Determine the magnitude of force in a truss member by determining the forces on a truss joint.

## Materials and Equipment

- Data collection system

- Building Better Bridges Kit, ME-3581

- Books, 4

- Paperclip

## Safety

Follow regular laboratory safety precautions.

## Procedure

### Part 1: Measuring Tension and Compression

- Select Sensor Data in SPARKvue and connect your sensor to your computing device.

- Connect two number 1 beams to the load cell. See Figure 1.

Figure 2: Beams connected to load cell

- Zero out the sensor and start collecting data.

- Apply a tension force by pulling the beams away from each other. Observe what happens to the data as you do this.

- Next, apply a compression force by pushing the beams towards each other. Again, observe what happens to the data as you do this.

### Part 2: Tension and Compression in Trusses

Tension and compression forces are created in truss members due to the joints pulling and pushing on the members. The force from the joints transfer to the member, which in turn transfers to another joint. The force on the members is equal and opposite to the forces on the joints, just as the force from the supports on a bridge are equal and opposite to the forces from the load.

- Place all of the weights on the hanger and measure the weight of the load with the load cell. Record your data in the diagram under Data Collection.

- Build the bridge shown in Figure 2. Support it with books at point B and C. You will need to have it high enough so that when you attach weights, they do not touch the table. Three books on each side should give you enough height. Start with the load cell placed between member (AD).

Figure 4: Bridge Construction and Set-up

- Zero out the sensor and then hang the weight from point D using a paperclip.

- Start collecting data. Measure and record the force in each member with the load cell in the diagram under Data Collection. Do not move the weights from point D. Move the load cell to each member you are measuring.

Note: Refer to activity 1. Forces in Equilibrium, if needed.

- Indicate whether a member is in tension or compression by drawing arrows on the member in the diagram under Data Collection.

## Data Collection

Tension and Compression Diagram

## Questions and Analysis

- In Part 1, you investigated how data was affected when you applied a compression force and a tension force to the load cell. What is different about the force measured by the load cell when a beam is in compression versus tension?

- Which members are under tension? Propose and explanation.

- What is the magnitude of the reaction support forces at point B and C? Write the values down on the diagram and justify your answer.

- Are the forces in the diagonal members greater or less then the support forces? Propose an explanation.

- In which member was the force approximately equal to the load?