# Investigation 12E: Designing Gear Machines

Essential question:  How do you make a large gear ratio?

How are gear machines designed?

Your challenge is to design machines with the following ratios. NOTE: One of these ratios cannot be built with the gears in your set.

Design, build, and test the following machines.

| Machine | Input turns | Output turns | NOTE: One of these four machines CANNOT be made with the gears you have. One of your challenges is to determine which machine cannot be built with your gears and propose an additional gear that would make that machine possible. |
| --- | --- | --- | --- |
| A | 4 | 1 |  |
| B | 6 | 1 |  |
| C | 10 | 1 |  |
| D | 9 | 4 |  |
| E | 12 | 1 |  |
| F | 18 | 1 |  |

- For each ratio that you can actually build fill in the numbers of teeth in the diagrams
on the next page.

- Write out the sequence of ratios that describes each machine and show that the ratios multiply together to give the overall ratio the design calls for.

- What additional gear would you need in order to build the ratio you could not make?

- Write out a series of ratios that would make this “un-built” machine if you had the
missing gear.